Developer Integration Guide & API Specification

Transit.dll provides hardware interfacing and serial communication routines for SDR devices. It is compiled for both 32-bit (x86) and 64-bit (x64) architectures using standard C export linkage.

Function Reference

1. AutoConnectSDR

Scans available serial ports and establishes a connection to a compatible SDR unit.

int __stdcall AutoConnectSDR(char* outBuffer, int maxLength);

- Parameters:

  - outBuffer (char*, [out]): Pre-allocated buffer where the status/connection message is written.

  - maxLength (int, [in]): Total allocated size of outBuffer in bytes (recommended: 255).

- Returns:

  - > 0: Successful connection. The returned integer represents the active COM port number (e.g., 3 for COM3).

  - 0 or -1: Connection failed; no compatible device found.

2. CheckConnection

Queries the hardware state to verify if the SDR is responsive and still actively connected.

void __stdcall CheckConnection(char* outBuffer, int maxLength);

- Parameters:

  - outBuffer (char*, [out]): Pre-allocated buffer to store the connection status text.

  - maxLength (int, [in]): Size of outBuffer in bytes.

- Returns: void

3. DisconnectSDR

Safely terminates the serial connection and releases the COM port.

void __stdcall DisconnectSDR();

- Parameters: None

- Returns: void

4. CommandTokens

Processes token-based instruction sends through the device controller.

void __stdcall CommandTokens(const char* inputStr, char* outBuffer, int maxLength);

- Parameters:

  - inputStr (const char*, [in]): Input token.

  - outBuffer (char*, [out]): Pre-allocated buffer to store the processed response.

  - maxLength (int, [in]): Size of outBuffer in bytes.

- Returns: void

5. SendCommandToSDR

Transmits a raw text command directly to the connected SDR hardware.

int __stdcall SendCommandToSDR(const char* inputStr);

- Parameters:

  - inputStr (const char*, [in]): Null-terminated command string to transmit

- Returns:

  - 1 (or 0): Transmission status code (returns non-zero on successful write).

All radio adjustment setting, commands are below and you can copy and paste it into your program.